# NAVIGATION TECH STACK

since we will be getting a fixed map and a instantaneous map

we will be using A* algo solely on the fixed map .

then we will use the spatio temporal voxel layer(it develops a 3d grid where each block danger level is classified by the hieght

if the height is too high or low it is ignored but if the hieght is similar to the rover , we classify as the danger and turn all such blocks into a 2D grid map) for the instantaneous map as it becomes something like a survival instinct for the rover , helps the rover react.

how to navigate the costmaps:

Forming the shortest path:

Depending on the steering style we will use either SmacPlanner2D or SmacPlannerHybrid plugin

both depend on the A* algorithm

what is A* alogrithm :

a shortest path deciding algorithm

it works by assigning the scores to all the paths forming on the criteria

shortest path

the distance from the obstacle.

then the algorithm will choose the path with the best score

tank style(Differential Drive): in this the path that will be suggested will with sharp 90 degree angles,as in the steering system the rover will drive exactly like a tank.thus we will use the SmacPlanner2D plugin

car Style(Ackermann) : in this the path that will be suggested will be including curve angles ,simulating how a car based rover will drive.thus we will use the SmacPlannerHybrid plugin

All these will work on both the rasberry pi and jetson 
the control algorithm will differ

How to control the rover to take the new path:

for this we can use Regulated Pure Pursuit(rpp)(preferrable for rasberry pi)

it keeps following the line set by the A* path 
it has a built in parameter “use_cost_regulated_linear_velocity_scaling”, which slows down the rover in case of a sharp turn or a obstacle

If we use use jetson,

we can also try the model predictive path integral(MPPI)(preferrable for jetson )

how does it work:

it  forms hundreds of simulated paths at once

then it brings in tiny variations in the speed to simulate skidding,or skinholes or any problem that might arise due to uneven terrain

then it will check the end of each path how the rover ended up

it will disregard the crashed rovers and pick the best path or the path where rover makes it safely

thus takes up a lot of compute power.